CITY OF SALMON ARM

BYLAW NO. 4656

# A bylaw to amend “District of Salmon Arm Zoning Bylaw No. 2303”

WHEREAS pursuant to the December of 2023 amendments of the Local Government Act [RSBC 2015] Chapter 1,  Council of the City of Salmon Arm is legislated to require enabling a secondary suite or an accessory dwelling unit with a single family dwelling across all residential lands outside of the Urban Containment Boundary;

AND WHEREAS as required by the Province of British Columbia, written submissions were accepted but no Public Hearing was held;

NOW THEREFORE the Council of the City of Salmon Arm in open meeting assembled enacts as follows:

- “District of Salmon Arm Zoning Bylaw No. 2303” is hereby amended as follows:

- by replacing the “Rural Detached Suite” in section 2.2 with “Rural Accessory Dwelling Unit” defined as follows:

Rural Accessory Dwelling Unit means a dwelling unit in any Agricultural “A” Zone that consists of a Z240 mobile home, an A277 modular home or a site-built structure and shall not include a travel trailer, recreational vehicle or storage container.

A Rural Accessory Dwelling Unit on a parcel within the ALR is subject to the Agricultural Land Commission Act regulations, including floor area limitations.

- by replacing section 34, the A-1 Agricultural Zone, with Schedule “A,” attached hereto and forming a part of this Bylaw;

- by replacing section 35, the A-2 Rural Holding Zone with Schedule “B,” attached hereto and forming a part of this Bylaw;

- by replacing section 36, the A-3 Small Holding Zone with Schedule “C,” attached hereto and forming a part of this Bylaw;

2. 	SEVERABILITY

If any part, section, sub-section, clause of this bylaw for any reason is held to be invalid by the decisions of a Court of competent jurisdiction, the invalid portion shall be severed and the decisions that it is invalid shall not affect the validity of the remaining portions of this bylaw.

3. 	ENACTMENT

Any enactment referred to herein is a reference to an enactment of British Columbia and regulations thereto as amended, revised, consolidated or replaced from time to time.

4. 	EFFECTIVE DATE

This bylaw shall come into full force and effect upon adoption of same.

5. 	CITATION

This bylaw may be cited as “City of Salmon Arm Zoning Amendment Bylaw No. 4656”

READ A FIRST TIME THIS		      	DAY OF 				 2024

READ A SECOND TIME THIS	 	DAY OF 				 2024

READ A THIRD TIME THIS	      	  	DAY OF				 2024

ADOPTED BY COUNCIL THIS   	   	DAY OF  				 2024

MAYOR

CORPORATE OFFICER

Schedule “A”

# SECTION 34 - A-1 - AGRICULTURE ZONE

Purpose

34.1	The A-1 Zone is intended to identify and preserve land with good agricultural or forestry potential.  Development on land located in the A-1 Zone will respect the rural nature of the area.

Regulations

34.2	On a parcel zoned A-1, no building or structure shall be constructed, located or altered and no plan of subdivision approved which contravenes the regulations set out in the A-1 Zone or those regulations contained elsewhere in this Bylaw.

Permitted Uses

34.3	The following uses and no others are permitted in the A-1 Zone:

.1	agriculture;

.2	single family dwelling;

.3	one secondary suite or one rural accessory dwelling unit (refer to ALC Act and ALR Regulations);

.4	bed and breakfast, limited to three let rooms in the principal dwelling;

.5	boarders, limited to two;

.6	equestrian facility;

.7	equestrian facility campsite;

.8	family childcare facility;

.9	group childcare;

.10	home occupation;

.11	intensive agriculture;

.12	silviculture;

.13	outdoor recreation;

.14	kennel;

.15	accessory use, including the retail sale of agricultural products produced on the parcel.

Maximum Number of Dwelling Units

34.4	The maximum number of dwelling units shall be two per parcel comprising of the principal use dwelling and one secondary suite or one rural accessory dwelling unit.

Maximum Residential Floor Area

34.5	The maximum gross floor area of the principal use single family dwelling with or without a secondary suite shall be no greater than 500 square metres (5,382 ft2) and of a rural accessory dwelling unit shall be no greater than 90 square metres (968 ft2).

Refer to the ALR Regulation for maximum residential total floor area and how this is calculated for parcels in the ALR.

Maximum Height of Residential Buildings

34.6	The maximum height of a residential building shall be 10 metres (32.8 feet).

Maximum Height of Non-Residential Buildings

34.7	The maximum height of accessory buildings, with or without a rural accessory dwelling unit, shall be 12 metres (39.4 feet).

Minimum Parcel Size

34.8	The minimum parcel size shall be 8 hectares (19.8 acres).

Minimum Parcel Width

34.9	The minimum parcel width shall be 150 metres (492.1 feet).

Minimum Setback of Non-Agricultural Use Principal and Accessory Buildings

34.10	The minimum setback of principal and accessory buildings intended to accommodate non-agricultural uses from the:

.1	Front parcel line shall be	6.0 metres (19.7 feet)

.2	Rear parcel line shall be	6.0 metres (19.7 feet)

.3	Interior side parcel line shall be	3.0 metres ( 9.8 feet)

.4	Exterior side parcel line shall be	6.0 metres (19.7 feet)

Refer to Section 4.9 for “Special Building Setbacks” which may apply.

Minimum Setback of Rural Accessory Dwelling Unit

34.11	The minimum setback of a rural accessory dwelling unit from all parcel lines shall be 6 metres (19.7 feet).

Minimum Setback of Agricultural Use Buildings or Structures

34.12	The minimum setback of buildings and structures intended to accommodate agricultural uses from the:

.1	Front parcel line shall be	30.0 metres ( 98.4 feet)

.2	Rear parcel line shall be	15.0 metres ( 49.2 feet)

.3	Interior side parcel line shall be	15.0 metres ( 49.2 feet)

.4	Exterior side parcel line shall be	30.0 metres ( 98.4 feet)

.5	Any single family dwelling shall be	15.0 metres ( 49.2 feet)

.6	Any watercourse or body of water shall be	30.0 metres ( 98.4 feet)

Refer to “Pound and Animal Control Bylaw” for special setbacks which may apply.

Minimum Setback of Kennels

34.13	The minimum setback of kennels from all parcel lines shall be 30 metres (98.4 feet).

Minimum Setback of Feed Lots

34.14	The minimum setback of feed lots from:

.1	any highway right-of-way shall be	100.0 metres  (328.1 feet)

.2	any parcel not zoned A-1 shall be 	65.0 metres  (213.3 feet)

Sale of Agricultural Products

34.15	The retail sale of agricultural products produced on the parcel is permitted provided the maximum floor area of the retail sale stand does not exceed 40 square metres (430.5 square feet).

Parking

34.16	Required off-street parking shall be as prescribed in Appendix I.

Schedule “B”

SECTION 35 – A-2 – RURAL HOLDING ZONE

Purpose

35.1	The A-2 Zone is intended to provide predominantly residential parcels and small-scale agriculture in a rural setting.

Regulations

35.2	On a parcel zoned A-2, no building or structure shall be constructed, located or altered and no plan of subdivision approved which contravenes the regulations set out in the A-2 Zone or those regulations contained elsewhere in this Bylaw.

Permitted Uses

35.3	The following uses and no others are permitted in the A-2 Zone:

.1	agriculture;

.2	single family dwelling;

.3	one secondary suite or one rural accessory dwelling unit (refer to ALC Act and ALR Regulations);

.4	bed and breakfast, limited to three let rooms;

.5	boarders, limited to two;

.6	equestrian facility;

.7	equestrian facility campsite

.8	family childcare facility;

.9	group childcare;

.10	home occupation;

.11	kennel;

.12	outdoor recreation;

.13	silviculture;

.14	accessory use, including the retail sale of agricultural products produced on the parcel.

Maximum Number of Dwelling Units

35.4	The maximum number of dwelling units shall be two per parcel comprising of the principal use dwelling and one secondary suite or one rural accessory dwelling unit.

Maximum Residential Floor Area

35.5	The maximum gross floor area of the principal use single family dwelling with or without a secondary suite shall be no greater than 500 square metres (5,382 ft2) and of a rural accessory dwelling unit shall be no greater than 90 square metres (968 ft2).

Refer to the ALR Regulation for maximum residential total floor area and how this is calculated for parcels in the ALR.

Maximum Height of Residential Buildings

35.6	The maximum height of a residential building shall be 10 metres (32.8 feet).

Maximum Height of Non-Residential Buildings

35.7	The maximum height of accessory buildings, with or without a rural accessory dwelling unit, shall be 12 metres (39.4 feet).

Minimum Parcel Size

35.8	The minimum parcel size shall be 4 hectares (9.9 acres).

Minimum Parcel Width

35.9	The minimum parcel width shall be 100 metres (328.1 feet).

Minimum Setback of Non-Agricultural Use Principal and Accessory Buildings

35.10	The minimum setback of principal and accessory buildings intended to accommodate non-agricultural uses from the:

.1	Front parcel line shall be	6.0 metres (19.7 feet)

.2	Rear parcel line shall be	6.0 metres (19.7 feet)

.3	Interior side parcel line shall be	3.0 metres ( 9.8 feet)

.4	Exterior side parcel line shall be	6.0 metres (19.7 feet)

Refer to Section 4.9 for “Special Building Setbacks” which may apply.

Minimum Setback of Rural Accessory Dwelling Unit

35.11	The minimum setback of a rural accessory dwelling unit from all parcel lines shall be 6 metres (19.7 feet).

Minimum Setback of Agricultural Use Buildings or Structures

35.12	The minimum setback of buildings and structures intended to accommodate agricultural uses from the:

.1	Front parcel line shall be	30.0 metres (98.4 feet)

.2	Rear parcel line shall be	15.0 metres (49.2 feet)

.3	Interior side parcel line shall be	15.0 metres (49.2 feet)

.4	Exterior side parcel line shall be	30.0 metres (98.4 feet)

.5	Any single family dwelling shall be	15.0 metres (49.2 feet)

.6	Any watercourse or body of water shall be	30.0 metres (98.4 feet)

Refer to “Pound and Animal Control Bylaw” for special setbacks which may apply.

Minimum Setback of Kennels

35.13	The minimum setback of kennels from all parcel lines shall be 30 metres (98.4 feet).

Sale of Agricultural Products

35.14	The retail sale of agricultural products produced on the parcel is permitted provided the maximum floor area of the retail sale stand is 40 square metres (430.5 square feet).

Parking

35.15	Required off-street parking shall be as prescribed in Appendix I.

Schedule “C”

# SECTION 36 - A-3 - SMALL HOLDING ZONE

Purpose

36.1	The A-3 Zone is intended to provide for the creation of two hectare parcels in areas specified in the Official Community Plan where further urbanization and servicing is not anticipated and where the Agricultural Land Commission (ALC has either excluded the area from the Agricultural Land Reserve or where the ALC has agreed to the A-3 Zoning).

Regulations

36.2	On a parcel zoned A-3, no building or structure shall be constructed, located or altered and no plan of subdivision approved which contravenes the regulations set out in the A-3 Zone or those regulations contained elsewhere in this Bylaw.

Permitted Uses

36.3	The following uses and no others are permitted in the A-3 Zone

.1	agriculture;

.2	single family dwelling;

.3	one secondary suite or one rural accessory dwelling unit (refer to ALC Act and ALR Regulations);

.4	bed and breakfast, limited to three let rooms;

.5	boarders, limited to two;

.6	family childcare facility;

.7	group childcare;

.8	home occupation;

.9	silviculture;

.10	accessory use, including the retail sale of agricultural products produced on the parcel.

Maximum Number of Dwelling Units

36.4	The maximum number of dwelling units shall be two per parcel comprising of the principal use dwelling and one secondary suite or one rural accessory dwelling unit.

Maximum Residential Floor Area

36.5	The maximum gross floor area of the principal use single family dwelling with or without a secondary suite shall be no greater than 500 square metres (5,382 ft2) and of a rural accessory dwelling unit shall be no greater than 90 square metres (968 ft2).

Refer to the ALR Regulation for maximum residential total floor area and how this is calculated for parcels in the ALR.

Maximum Height of Residential Buildings

36.6	The maximum height of a residential building shall be 10 metres (32.8 feet).

Maximum Height of Non-Residential Buildings

36.7	The maximum height of accessory buildings, with or without a rural accessory dwelling unit, shall be 12 metres (39.4 feet).

Minimum Parcel Size

36.8	The minimum parcel size shall be 2 hectares (4.9 acres).

Minimum Parcel Width

36.9	The minimum parcel width shall be 50 metres (164.0 feet).

Minimum Setback of Non-Agricultural Use Principal and Accessory Buildings

36.10	The minimum setback of principal and accessory buildings intended to accommodate non-agricultural uses from all parcel lines shall be 6 metres (19.7 feet).

Refer to Section 4.9 for “Special Building Setbacks” which may apply.

Minimum Setback of Agricultural Use Buildings or Structures

36.11	The minimum setback of buildings and structures intended to accommodate agricultural uses from the:

.1	Front parcel line shall be	30.0 metres (98.4 feet)

.2	Rear parcel line shall be 	15.0 metres (49.2 feet)

.3	Interior side parcel line shall be	15.0 metres (49.2 feet)

.4	Exterior side parcel line shall be	30.0 metres (98.4 feet)

.5	Any single family dwelling shall be	15.0 metres (49.2 feet)

.6	Any watercourse or body of water shall be	30.0 metres (98.4 feet)

Refer to “Pound and Animal Control Bylaw” for special setbacks which may apply.

Sale of Agricultural Products

36.12	The retail sale of agricultural products produced on the parcel is permitted provided the maximum floor area of the retail sale stand is 40 square metres (430.5 square feet).

Parking

36.13	Required off-street parking shall be as prescribed in Appendix I.